# 🌡 Temperature Monitor using DS18B20 & 16x2 I2C LCD

## 📌Task  Overview

This project reads temperature from a DS18B20 digital sensor, displays the current temperature on a 16x2 I2C LCD, and logs the readings to the Serial Monitor.

The system uses 1-Wire communication to obtain accurate digital temperature readings and updates every second.

## 🧰 Components Used

- Arduino UNO

- DS18B20

- 16x2 I2C LCD (address 0x27)

- 4.7kΩ resistor (pull-up for DS18B20)

- Jumper wires

## 🔌 Wiring Diagram

### 🔹 DS18B20 Connections

| DS18B20 Pin | Connect To |
| --- | --- |
| 1 (GND) | GND |
| 2 (DATA) | D2 |
| 3 (VCC) | 5V |

⚠ Important:
Connect a 4.7kΩ resistor between D2 and 5V (pull-up resistor).

### 🔹 I2C LCD Connections

| LCD Pin | Arduino UNO |
| --- | --- |
| VCC | 5V |
| GND | GND |
| SDA | A4 |
| SCL | A5 |

## ⚙ How It Works

- The DS18B20 measures temperature digitally.

- Arduino communicates using the OneWire protocol.

- The temperature value is retrieved in Celsius.

- The value is:

  - Displayed on the 16x2 LCD

  - Printed to the Serial Monitor

- The system updates every 1 second.

## 💻 Arduino Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#define ONE_WIRE_BUS 2   // DS18B20 connected to D2

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

LiquidCrystal_I2C lcd(0x27, 16, 2);

void setup() {

Serial.begin(9600);

sensors.begin();

lcd.init();

lcd.backlight();

lcd.setCursor(0,0);

lcd.print("Temp Monitor");

delay(1500);

lcd.clear();

}

void loop() {

sensors.requestTemperatures();

float tempC = sensors.getTempCByIndex(0);

lcd.setCursor(0,0);

lcd.print("Temp: ");

lcd.print(tempC,1);

lcd.print((char)223);

lcd.print("C   ");

Serial.print("Temperature: ");

Serial.print(tempC);

Serial.println(" C");

delay(1000);

}